Case study: AT3545 laser

Subject: BER Prediction based on Laser Bias and Laser Module Features

Authors: Morteza Ziyadi

Date: 03/17/2017

Detail:

In this report, we aim to predict the BER based on different features of the laser module and laser bias. We use the following features for this purpose. It should be note that this analysis is run on the 943 laser modules that 87 units are failed the BER. Also, in previous analysis we just considered the effect of SMSR, CNR, CSO, and CTB, but, in this report we look into all other features.

'Laser SMSR','CNR_61-laser', 'CNR_745-laser','CSO_Minus_61-laser', 'CSO_Plus_547-laser', 'CTB_295-laser',

'WaveLength-laser','ThresholdCrrt-laser', 'SlopEff-laser', 'LsrDistortion-laser', 'Attenuation_Raw-laser',

'Result Laser Bias mA(S21)',

'Result Ave db(S21)', 'Result digipad count(Distortion)',

'CNR_55_25', 'CNR_77_25', 'CNR_205.25',

'MER_741', 'MER2','BER_999'

Target: ‘BER_test'

The first analysis, we take a look at the histograms of the different features. Following is the result.

As we see, most of the lasers that failed have lower SMSR values compare to the passed units. CNR-745 is also a little bit skewed to the left for the failed units. It is interesting to note that CSO547 feature is high even for the failed units. It also seems like that the laser bias distribution is very similar for the passed and failed units. It could also make sense that some of the failed units have small MER values compare to the passed units. In the transmitter side, CNR values are also skewed to the lower values for the failed units. Now, let’s use random forest to see which of the features give more information regarding the BER test. At this step, we ignore the  'MER_741', 'MER2','BER_999' since they are just somehow present the BER test results and are the final measurements.

| Feature | Feature Importance Value |
| --- | --- |
| Laser SMSR | 0.299339872 |
| WaveLength-laser | 0.095235411 |
| SlopEff-laser | 0.080318554 |
| CSO_Plus_547-laser | 0.070718063 |
| CNR_205.25 | 0.069868844 |
| CNR_55_25 | 0.069244658 |
| Result Ave db(S21) | 0.05027154 |
| CNR_77_25 | 0.047552873 |
| Result Laser Bias mA(S21) | 0.032725861 |
| ThresholdCrrt-laser | 0.031699015 |
| CTB_295-laser | 0.030854107 |
| CNR_745-laser | 0.026675522 |
| Result digipad count(Distortion) | 0.025136019 |
| CSO_Minus_61-laser | 0.022720245 |
| LsrDistortion-laser | 0.022133346 |
| CNR_61-laser | 0.017113353 |
| Attenuation_Raw-laser | 0.008392716 |

This one shows that Laser SMSR, wavelength and slopeff play an important role in BER test results. It could also be mentioned that the laser bias data doesn’t show strong impact on the BER, however, we know Slopeff effects the laser bias. It could be interpreted as the measured laser bias data doesn’t provide enough information on the BER test however, physically it impacts the BER test and its impact comes from the Slopeff feature. Also, CSO547 and CNR measurements at the transmitter side have the next important impact on the BER test results. In the following we try to use clustering to find out if there is any specific feature that can be used to separate the failed units and passed units or not.

If we run the clustering on our dataset, we get the following result.

| cluster | number of Passed | number of Failed | Failure Percentage |
| --- | --- | --- | --- |
| 0 | 117 | 6 | 4.87 % |
| 1 | 222 | 23 | 9.38 % |
| 2 | 120 | 12 | 9.09 % |
| 3 | 145 | 8 | 5.2 % |
| 4 | 243 | 34 | 12.27 % |

It should be mentioned that Total Failure Percentage = 8.92%.

As the result shows, clusters 0 and 3 have lower failure percentage. By looking at the clusters centers for each feature we might get some insight about the specific detail on these two clusters. Following is the result for each cluster’s center.

|  | Cluster 0 Center | Cluster 1 Center | Cluster 2 Center | Cluster 3 Center | Cluster 4 Center |
| --- | --- | --- | --- | --- | --- |
| Laser SMSR | 39.5602 | 39.6691 | 40.3642 | 41.0461 | 40.4434 |
| CNR_61-laser | 52.6715 | 52.8600 | 52.7394 | 52.7118 | 52.7819 |
| CNR_745-laser | 50.3496 | 50.4829 | 50.3591 | 50.3451 | 50.4025 |
| CSO_Minus_61-laser | 57.1390 | 56.1898 | 56.1258 | 55.7850 | 56.2480 |
| CSO_Plus_547-laser | 48.0415 | 47.1024 | 47.5394 | 47.2373 | 47.1310 |
| CTB_295-laser | 69.2878 | 69.5576 | 68.6068 | 69.1882 | 69.0736 |
| WaveLength-laser | 1538.9291 | 1556.3891 | 1538.5032 | 1556.5489 | 1556.6485 |
| ThresholdCrrt-laser | 16.1829 | 16.9633 | 14.7326 | 16.3144 | 15.3643 |
| SlopEff-laser | 0.2345 | 0.2265 | 0.2201 | 0.1979 | 0.2213 |
| LsrDistortion-laser | 58.0724 | 57.1122 | 56.9644 | 56.6222 | 57.1711 |
| Attenuation_Raw-laser | 7.9451 | 7.8969 | 7.7216 | 7.8840 | 7.7175 |
| Result Laser Bias mA(S21) | 75.5874 | 75.7472 | 78.4454 | 83.7214 | 76.1821 |
| Result Ave db(S21) | -21.3543 | -21.3583 | -21.4039 | -21.4441 | -21.3611 |
| Result digipad count(Distortion) | 13.3984 | 15.6735 | 3.3864 | 2.0784 | 7.1733 |
| CNR_55_25 | 53.7619 | 53.7858 | 54.3417 | 53.9814 | 53.8692 |
| CNR_77_25 | 53.7580 | 53.5487 | 54.1522 | 53.7238 | 53.6066 |
| CNR_205.25 | 53.9495 | 53.6690 | 54.3292 | 54.2037 | 53.7290 |

To have a better insight for these numbers, we can use a bar plot to see all of them. Following is the same numbers in bar plot formats.

If we look at the clusters 0 and 3 ‘s centers, we don’t see a specific pattern repeated for these two clusters that is different than the other cluster centers. It seems like that there is no correlation between clusters centers and the failure percentage. But there is a point to mention out of this result and it is for cluster 3. This cluster has a smallest center value for Slopeff which leads to highest laser bias and lowest digipad_count(distortion). We knew the first relation between the slopeff and laser bias. For the second one, it is intersing that the lasers which have higher bias in the transmitter they have smallest value for distortion measure at the transmitter station.

In the following, we investigate on SMSR value. Based on the histograms, we found that SMSR value for the failed units is lower compare to the passed units. If we use the threshold of 35 for SMSR, we get the following results.

- Number of Passed Units with SMSR<35 = 48

- Number of Failed Units with SMSR<35 = 31

- Failure percentage for SMSR<35 = 40.24 %

- Compare to 9.22% failure rate in total of the units (87 failed out of 943), we have a large number of failed units in SMSR<35.

If we plot the histogram of the units with SMSR<35, we have the following results:

Now, if we only consider the units with SMSR>=35, we get the following statistics:

- Number of Passed Units with SMSR>=35 = 808

- Number of Failed Units with SMSR>=35 = 56

- Failure percentage for SMSR>=35 = 6.48 %

- Compare to 9.22% failure rate in total of the units (87 failed out of 943), we can lower down the failure rate by 3% only by filtering the SMSR values.

If we plot the histogram of the units with SMSR>=35, we get the following results:

Now, if we run the machine learning algorithm to find out the important features on the units with SMSR>=35 to predict the BER test result, we get the following values:

| Feature | Feature Importance Value |
| --- | --- |
| WaveLength-laser | 0.171043233 |
| Laser SMSR | 0.139026982 |
| Result Ave db(S21) | 0.093513295 |
| CNR_55_25 | 0.090780872 |
| Result Laser Bias mA(S21) | 0.078802025 |
| CSO_Plus_547-laser | 0.078118796 |
| CNR_205.25 | 0.075098259 |
| ThresholdCrrt-laser | 0.047792737 |
| CNR_77_25 | 0.045711407 |
| SlopEff-laser | 0.037513397 |
| Result digipad count(Distortion) | 0.028163511 |
| Attenuation_Raw-laser | 0.024418957 |
| CTB_295-laser | 0.024263561 |
| CSO_Minus_61-laser | 0.022021192 |
| CNR_61-laser | 0.018502661 |
| LsrDistortion-laser | 0.015301248 |
| CNR_745-laser | 0.009927868 |

This result shows that wavelength of the laser alongside with the SMSR play important role. It is interesting that, the slopeff which was an important factor for all the units for the units with SMSR>=35 is not that much important. Also, if we look at the Wavelgnth histogram, we observe that units with wavelength >=1560 nm are very dominant in failed units. If we filter by this feature, we get the following result:

- Number of Passed Units with SMSR>=35 and Wavelength>=1560 = 108

- Number of Failed Units with SMSR>=35 and Wavelength>=1560 = 18

- Failure percentage for SMSR>=35 and Wavelength>=1560 = 14.28 %

- Compare to 6.48 % failure rate in all the units with SMSR>=35 (56 failed out of 864), we have higher number of failed units with Wavelength>=1560.

Following is the histogram of the units with SMSR>=35 and Wavelength>=1560.

It is very interesting that the filed units inside this category, they have laser bias of <76 mA. And almost all the others than have high laser bias passed the BER test. (see the laser bias histogram)

For the units with SMSR>35 and Waveelngth<1560, we get the following results:

- Number of Passed Units with SMSR>=35 and Wavelength<1560 = 700

- Number of Failed Units with SMSR>=35 and Wavelength<1560 = 38

- Failure percentage for SMSR>=35 and Wavelength<1560 = 5.14 %

- Compare to 6.48 % failure rate in all the units with SMSR>=35 (56 failed out of 864), we can lower down the failure by 1.5% .

Following is the result for the units with SMSR>=35 and Wavelength<1560.

And now if we run the machine learning algorithm to find the feature importance on the units with SMSR>=35 and Wavelength<1560, we get the following result:

| Feature | Feature Importance Value |
| --- | --- |
| Laser SMSR | 0.198835753 |
| CNR_55_25 | 0.114828572 |
| CSO_Plus_547-laser | 0.104318323 |
| CNR_77_25 | 0.065984443 |
| Result Ave db(S21) | 0.06480284 |
| CNR_205.25 | 0.063336006 |
| SlopEff-laser | 0.05738905 |
| Result digipad count(Distortion) | 0.043747396 |
| Attenuation_Raw-laser | 0.043728521 |
| WaveLength-laser | 0.043324115 |
| Result Laser Bias mA(S21) | 0.041730336 |
| ThresholdCrrt-laser | 0.031397329 |
| CTB_295-laser | 0.030280084 |
| CNR_745-laser | 0.028586609 |
| LsrDistortion-laser | 0.02560533 |
| CNR_61-laser | 0.023284654 |
| CSO_Minus_61-laser | 0.01882064 |

We see that laser SMSR, and CSO547 are important as well as CNR55.25 of the transmitter. By looking at the histogram of these feature we can find out there they are skewed for the failed units compare to the passed one.

In conclusion, SMSR and Wavelength could be use as initial features to investigate the failed units in more detail. Also, after filtering these features, it might be helpful to run more analysis and by using more data to investigate the possible reasons for failure. This report considers a roadmap of how we can investigate the failure results step by step. Making a decision will be made after more investigations and tests. It should be mentioned that gathering more data appropriately would help a lot in the analysis. Having data with lack of accuracy would lead to wrong decisions at the end. Furthermore, the data set that we used so far shows that the failure result depends less on the laser bias. In fact, it depends on laser bias after some filtering on SMSR. So, to achieve better prediction on laser bias, we would need to run other tests and measure other data such as full BER curve versus the optical power. Otherwise, using only the current laser bias data which is measured only by achieving the optical power wouldn’t help. Also, considering the physics of the lasers and BER test would help a lot in order to define new experiments. Using data analysis, we found different correlation that by combining with the physics of the phenomena we can have better and more accurate predictions.